# Packet Tracer - Use Cisco IOS Show Commands

# Objectives

Use Cisco IOS show commands

# Background / Scenario

The Cisco IOS show commands are used extensively when working with Cisco equipment. In this activity, you will use the show commands on a router located at an ISP.

# Instructions

## Connect to the ISP Cisco 4321 router.

In this part, you will use the terminal emulation software on ISP PC to connect to the Cisco 4321 router.

Click ISP PC.

Click Desktop tab. Select Terminal. Review the terminal configuration and click OK to continue.

The ISPRouter > prompt indicates that you are in user EXEC mode. Press Enter if the prompt did not display.

## Explore the show commands.

Use the information displayed by these show commands to answer the following questions.

### Explore show commands in User EXEC mode.

Open configuration window

Type show ? at the prompt.

#### Question:

List a few more show commands that are available in the user EXEC mode.

Enter show arp at the prompt.

#### Question:

Record the MAC address and the IP address listed.

- Enter show flash at the prompt.

#### Question:

Record the IOS image listed:

Enter show ip route at the prompt.

#### Question:

How many routes are listed in the table?

Enter show interfaces at the prompt.

#### Question:

Which interface is up and running?

| Interface | Status | Protocol |
| --- | --- | --- |
| GigabitEthernet 0/0/0 | Up | Up |
| GigabitEthernet 0/0/1 | Down | Down |
| Serial0/1/0 | Down | Down |
| Serial0/1/1 | Down | Down |

Enter show ip interface at the prompt.

#### Question:

According to the show ip interface output, which interface is connected?

Enter show version at the prompt.

#### Question:

What technology package is enabled currently on the router?

- Enter show protocols at the prompt.

#### Question:

Which protocols are enabled currently on the router?

Enter show running-config at the prompt.

#### Question:

What is the output?

Type your answers here.

### Explore the show commands in privileged EXEC mode.

Enter enable at the prompt to enter the privileged EXEC mode.

#### Question:

List some additional show commands in this mode.

Type your answers here.

Enter show running-config at the prompt.

#### Question:

What is the output?

Close configuration window

# Answer Key

  - Connect to the ISP Cisco 4321 router.

  - Explore the show commands.

    - Explore show commands in User EXEC mode.

List a few more show commands that are available in the user EXEC mode.

Answers will vary. These are a few of available show commands: show cdp, show class-map, show clock

Record the MAC address and the IP address listed.

Answers may vary. 0001.96CD.2501: 209.165.201.1, 000A.F3C7.5CD3: 209.165.201.5, and 0002.162C.67A7: 209.165.201.10

Record the IOS image listed:

isr4300-universalk9.03.16.05.S.155-3.S5-ext.SPA.bin

How many routes are listed in the table?

2

Which interface is up and running?

| Interface | Status | Protocol |
| --- | --- | --- |
| GigabitEthernet 0/0/0 | Up | Up |
| GigabitEthernet 0/0/1 | Administratively Down | Down |
| Serial0/1/0 | Down | Down |
| Serial0/1/1 | Down | Down |

According to the show ip interface output, which interface is connected?

GigabitEthernet0/0/0

What technology package is enabled currently on the router?

ipbasek9 and securityk9

Which protocols are enabled currently on the router?

Internet Protocol routing is enabled. Line protocol is up for GigabitEthernet0/0/0.

What is the output?

ISPRouter> show running-config

^

% Invalid input detected at '^' marker.

    - Explore the show commands in privileged EXEC mode.

List some additional show commands in this mode.

Answers will vary. Some example of the show commands are: show aaa, show access-list, show file

What is the output?

The output displays the current configurations and settings.

End of document